# MapShareVic

## Overview

#### Where things exist in Victoria are important facts which help us understand the way things are and help us make informed decisions about future places and management. MapshareVic holds a wealth of spatial data and information which will enable you to compile authoritative maps and tabular data on many themes in Victoria.

### Specifications

#### You may see a different view depending if you are browsing using a Desktop, Tablet or Handheld device.

#### Many factors can influence the performance of this application on your viewer

#### We recommend

- minimum 10 Mbps network

- Chrome browser

- The latest versions of the browsers should be used.  If old incompatible versions of Internet Explorer, Firefox or Chrome are being used there will be a reduction in performance

Contents

MapShareVic	1

Overview	1

Specifications	1

Basics	3

Desktop Viewer	3

Home Panel View	3

Tablet Viewer	4

Handheld Viewer	4

Search	5

Global Search	5

Search 1 – Zooms to selected option	5

Search 2 – Displays search results	6

Find Location tool	8

Layer Panel View	9

Layer Action menu	10

Panel Action Menu	11

Layer Panel Action Menu	11

Results Panel Action Menu -	11

Right Click on Map Menu	11

Printing	12

Tablet View Help	13

Display data on the map, when you can’t see it	14

Map Projections	15

Layer name is grey and is not displayed	16

How to measure Areas quickly	19

Change the look of data	20

How to filter your data in the map view	22

FAQ and Known Problems	23

Frequently asked questions	23

Known Problem	25

## Basics

### Desktop Viewer

#### Below are the main components to MapshareVic: using a Desktop Viewer

#### Try each of these components, get familiar with how they work and what they do

Select the XY option to expand the coordinates options for the curser position

Projections

GDA94   Latitude/Longitude    VicGrid 94  X/Y      MGA Zone 54 and 55 East/North

MGRS  Military Grid Reference System ( not visible since 25 June 2018 upgrade )

## Home Panel View

The Welcome page or default Panel view is Home, which will give you general web application information, including contact details

### Tablet Viewer

### Handheld Viewer

Toolbars and options are hidden for the best view of the map

## Search

### Global Search

The Global Search box allows you to search for address/spi/localities/features on the map and can be used in 2 ways, for an immediate zoom to a selection or with a click of the enter button the results will be displayed in a Results Panel View for further selection.

- Search – suggestions given          2. Search – results shown on left panel

for selection

### Search 1 – Zooms to selected option

Address   try [ 123 Smith ]  [ 123 Smith Port ]

Parcel SPI (Lot/Plan Number)  try [ 1\LP123 ]   Pnumber try [ P123 ]

Street  try [ Smith ] [ Smith C ]  [Smith Cre ]

Lot on Street  try [ Lot 4 Smith St ] [ Lot 1 Smith Ave ]

Locality  try [ Wunghnu ] [ Lady ]  Council  try [ Moyne ]

Postcode try [ 321 ]

Parish try [ Ballarat ]  [ Mildura ]

Crown Allotment  try [ Ballarat 15 1a ] allot 15 sec 1a [ Ballarat J ] sec J

Property Number  try [ 12345 Casey ]

### Search 2 – Displays search results

When you type a word into the Search box and select the Enter button, the results start to display in the results list immediately. More items display in the Results List as they are found.

These are now grouped for easier selection of numerous results, click the arrow

Results. Click on one of the search results and the map will reposition to its location.

Tablet View

### Find Location tool

Select the Find Location button in the Toolbar

The options will change depending which Theme you are currently in

For Forest Explorer Theme the following options are available

Select or Enter an option and then the OK button and the map will automatically zoom to the location

## Layer Panel View

Layer Control This panel displays the spatial layer list.

From here you can use the check boxes to select the layers you want shown on your map. Some layers have additional Layer action tools to allow you to see the Metadata about the layer.

Below are the components

Expand and Contract Layer lists with the  +  and  -  options

Turn Layers on/off with the Checkboxes.

Use the Transparency slider to view layers below.

Access additional tools with the Panel Action and Layer Action Menus

- Layer Panel Action Menu, displays the legend

Note : Only the Layers with a tick will be displayed in the legend

Check the current Theme being displayed on the map

### Layer Action menu

Select the > at the right of the Layer Name to display the Layer Action menu shown below

Zoom to full extent

Zoom the map to the full extent of the layer.

Zoom to visible scale

Zoom the map to a scale where the layer is visible.

Turn on/off layer visualizations

Create and view custom layer visualisations such as Heat Maps, Clustering and Layer Styles.

- For an outline only visualization, select white for the fill and then max the transparency

Toggle labels

Turn the layer’s labels on or off.

Customize labels

Toggle and customise the layer’s labels.

Metadata link

Click this Metadata link to view the Metadata about the layer.

## Panel Action Menu

Depending on the Panel View you have active, different menus will appear

### Layer Panel Action Menu

### Results Panel Action Menu -

## Right Click on Map Menu

## Printing

There are a number of different Print templates for you to choose from:

Layout –

A4 Landscape – DELWP, A3 Landscape – DELWP, A4 Portrait – DELWP, A3 Portrait – DELWP, A4 Portrait – Basic Style, A4 Portrait – Simple

Output format –

You can choose a number of different

Output formats – default is PDF, Rtf, Tiff, BMP, Gif, Jpeg, Png, GeoTIFF.

Resolution –

You can choose between

Low (96dpi) – Default, Medium (144 dpi), High (216 dpi) or Quality(360dpi).

Grid –

You can choose to print your map with a Grid.

None is the default, or choose: VicGrid Latitude/Longitude MGA Zone 54 MGA Zone 55

Map Scale –

Current scale is the default, or you can choose Current Extent, or a range of scales based on the Base Map scales.

Title –

You can add a Title to your map.

Lock print preview with map –

You can see exactly where the print extent will be with this ticked.

## Tablet View Help

Search for a location

Select Result in the left panel, and the Map will zoom to this location

Adjust options for best view of Map

### Display data on the map, when you can’t see it

Expand the   for the Group of Layers you wish to see on the map, then select the  arrow

On the next Menu, select   Zoom to visible scale

Crown Land Tenure- All Tenure Outline data is now displayed

If you still can’t see the data then check for known problems

## Map Projections

The curser position can be shown is several different projections

Select the XY option to expand the coordinates options for the curser position

Projections

GDA94   Latitude/Longitude    VicGrid 94  X/Y      MGA Zone 54 and 55 East/North

Military Grid Reference System (MGRS)

- The Geocentric Datum of Australia 1994 (GDA94) is Australia's official geodetic datum.

- The VicGrid94 projection has been chosen as the projection of choice as it is the most accurate projection with minimal distortions for the state of Victoria mapping applications.

- The Map Grid of Australia 1994 (MGA) is the standard map projection associated with GDA94.

- The Military Grid Reference System  (MGRS) is the geocoordinate standard used by NATO militaries for locating points on the earth.

## Layer name is grey and is not displayed

Some Layers will only display at certain scale range, this is to optimise the speed of the display of the map for the user.

If the Layer name is grey, then this data is not displayed even if it is ticked, because it is outside its scale range.

To display the data on the map from the layer name in grey italics

Select the   >   on the required layer, Contour 70-100K and select the Zoom to visible scale option which will zoom the map to 1:100,000

The Layer name changes to black

Contours are displayed at 1:100,000 using the Layer Contour 70-100K , which is now black

Select the   >    on the required layer, Parcel,

and select the Zoom to visible scale option which will zoom the map to 1:25,000

Parcels are displayed at 1:25,000 and the Layer name Parcel is now black

The contours are also displayed, using the Contour 1-70K, which is black

## How to measure Areas quickly

- Select the required feature with the Identify tool

- Select the Panel Action Menu in the upper right hand side of the panel

- Select the Measurement option

## Change the look of data

The capability to style the vector layers containing lines, points or polygons is available  - including parcel boundaries as shown in the image below...

Select the   >    on the required layer,

and then Turn on/off layer Visualizations

Options available

For a transparent fill choose the opacity required in the Colour option

Simply select None to go back to the default style

## How to filter your data in the map view

You can filter any data layer using the fields available to display the information you require

The filter tool is located in the Advanced tab, see the example below to show how it works

- To display only the Wildfire History after 2010 select the Advanced tab, then Filter tool

- In the form that appears, select the Layer Wildfire History, Season > 2010 as shown below.

- You can see that only the Wildfires that occurred after 2010 is now displayed.

- To return to the normal display, just select the Clear button

Map View of Wildfire Layer with Filter of all data after 2010

Map View of Wildfire Layer with no Filter

## FAQ and Known Problems

### Frequently asked questions

#### How do I display the Legend

- Select the button   at the top right hand corner of the Layer Panel

- Select Show Legend

- The Legend Panel will display showing only those layers that are currently visible

Or select the Show Legend option from the I want to... menu

Or select the Show Legend toolbar button on the Tools tab

#### How do I add my exported data to Avenza

Since the Georeferenced option is not available for exporting as a pdf.

Avenza PDF Maps consumes GeoTIFFs better than GeoPDFs. The software when consuming a GeoPDF renders it to a GeoTIFF.

In MapShareVic, export the image as a GeoTIFF, (which does not require the Georeferenced option ticked, as by its format it is automatically georeferenced)

Load into your Avenza PDF Maps.

#### How do I convert Co-ordinates

- Select the  XY   co-ordinates at the bottom right corner of map

- Different Projection co-ordinate  information will appear

To convert coordinates use the following online resource

http://tool-online.com/en/coordinate-converter.php

#### Vicmap Aerial Imagery not displaying

This basemap does not display below 1:800 scale. This data is made available for us to use as is, and we can not change the display properties.

1:800 und below

Change the scale to 1:800 and the Imagery will display

1:800 and above

### Known Problem

#### The data is not displaying

There is a known problem that when displaying more than 7 layers in the same mapservice, data disappears.

If you are having problems displaying data, you may require to untick/tick the layers until only a maximum of 6 layers in single folder are ticked or until the data required is displayed on the map.